# Optimised Robot — Design Documentation

Table of Contents

# Optimised Robot — Design Documentation

Course: Intelligent Systems and Robotics Laboratory (ISRLAB)
A.Y.: 2025/26
Supervisor: Prof. Giovanni De Gasperis

Team name: Robotics Mhackeroni

Member Names:

Abid Hossain

Valerio Agostini

Lorenzo di Giovanni

Luca Mazzoni

## Table of Contents

- General Project Description

- Robot Model & Technical Description

- Simulator Environment

- Robot Goal Definition

- Agent Architecture Design

- Testing Protocol

- Experimental Results

- UML Diagrams

- Non-Functional Requirements

## 1. General Project Description

### Abstract

This project presents the design and implementation of an autonomous differential-drive mobile robot simulated in CoppeliaSim. The robot is engineered to execute a sequence of complex navigation tasks, including precise line following, roundabout navigation via colour-coded markers, dynamic obstacle avoidance using proximity sensors, and an object pickup-and-return mission. A hybrid deliberative-reactive control architecture, governed by a Behaviour Tree, is employed to manage task arbitration and ensure robust real-time performance.

### Project Description

Optimised Robot is an autonomous mobile robot agent simulated in CoppeliaSim (formerly V-REP). The robot operates on a predefined closed-loop track and must complete a series of increasingly complex tasks without human intervention. The control architecture is built around a Behaviour Tree (BT) that governs high-level decision making, dispatching commands to low-level motor and sensor routines.

The robot demonstrates five integrated capabilities:

| # | Capability | Description |
| --- | --- | --- |
| 1 | Line Following | Tracks a black line on the floor using five vision sensors |
| 2 | Roundabout Navigation | Detects coloured markers and executes a choreographed 4-step steering sequence through a roundabout |
| 3 | Obstacle Avoidance | Detects bowl-shaped obstacles via proximity sensors and bypasses them using bang-bang wall-following |
| 4 | Object Pickup & Delivery | Identifies a dark-blue marker (pickup point) and a red marker (drop-off point), then executes a U-turn to return the object |
| 5 | Vocal Command Response | Receives broadcasted stop/start/mode-switch messages and updates the behaviour tree blackboard accordingly |

The project showcases a reactive, deliberative hybrid architecture: low-level reflexes handle sensor-driven immediate response, while the behaviour tree provides goal-oriented deliberation.

## 2. Robot Model & Technical Description

### 2.1 Model Origin

The robot is a custom-assembled differential-drive mobile robot built within the CoppeliaSim model library. It is a two-wheeled ground vehicle with a passive caster for balance.

### 2.2 Body Shape

The robot has a compact rectangular chassis. Two motorised wheels are mounted symmetrically on the lateral axis; the chassis sits low to the ground to keep all five downward-facing vision sensors within effective focal range of the track surface.

### 2.3 Sensor Set

#### Vision Sensors (×5) — Downward-facing binary image sensors

| Sensor Name | Position | Purpose |
| --- | --- | --- |
| LeftSensor | Front-left | Detects line left deviation |
| MiddleSensor | Front-centre | Detects line centre; reads RGB colour patches for mission logic |
| RightSensor | Front-right | Detects line right deviation |
| InternalLeftSensor | Inner-left | Fine-grained correction for mild left drift |
| InternalRightSensor | Inner-right | Fine-grained correction for mild right drift |

Each sensor runs a Lua sysCall_vision callback with simVision.binaryWorkImg to threshold the image. The binary result (index [10] of the packed packet, representing mean pixel intensity) is read by readsensor.py and broadcast on the simulation message bus.

Binary threshold parameters (all sensors):

Intensity range : 0.50 – 1.00
Hue range       : 0.10 – 0.50  (picks up dark/black vs white)
Saturation range: 0.50 – 1.00
Angle           : 0.00 – 1.5710 rad

#### Proximity Sensors (×3) — Ray-based distance sensors

| Sensor Name | Position | Threshold |
| --- | --- | --- |
| CentralProximitySensor | Front-centre | 0.20 m |
| LeftProximitySensor | Front-left | 0.20 m |
| RightProximitySensor | Front-right | 0.20 m |

### 2.4 Actuator Set

| Actuator | Joint Name | Type |
| --- | --- | --- |
| Left wheel motor | DynamicLeftJoint | Continuous revolute joint |
| Right wheel motor | DynamicRightJoint | Continuous revolute joint |

Control is velocity-based: sim.setJointTargetVelocity() is called each actuation tick with a scaled value computed as speed × dt.

### 2.5 Calibration Data

| Parameter | Value |
| --- | --- |
| Base movement speed (rad_mov) | π rad/s |
| Base rotation speed (rad_rot) | π rad/s |
| Wall-following base speed (wf_base_speed) | 3.5 units |
| 180° turn angular velocity | 14.0 rad/s |
| 180° turn duration | π / 14.0 ≈ 0.224 s |
| Roundabout micro-steer duration | 0.35 s |
| Colour cooldown (roundabout) | 1.0 s |
| Proximity detection threshold | 0.20 m (all three sensors) |

### 2.6 Degrees of Freedom

- DOF: 2 (left wheel velocity, right wheel velocity)

- Movable parts: 2 motorised wheels

- The robot achieves all motion primitives (forward, rotate-in-place left/right, differential steer) through combinations of these two velocity commands.

## 3. Simulator Environment

Simulator: CoppeliaSim (EDU) — chosen per the IEEE survey of robotics simulators for its:

- Native Python scripting via embedded interpreter

- Accurate physics engine (Bullet/ODE)

- Vision sensor support with image processing callbacks

- Built-in graph/plotting for real-time sensor data visualisation

- Message broadcasting system enabling a decoupled multi-script architecture

Environment layout:

- Closed-loop black line track on a white floor

- One roundabout section with green (left entry marker) and yellow (right entry marker) coloured patches

- Bowl-shaped rigid-body obstacles placed on the track

- Dark-blue rectangular marker (pickup activation point) — placeable on track

- Red rectangular marker (drop-off / return trigger point) — placeable on track

## 4. Robot Goal Definition

### 4.1 Main Goal

Complete a full autonomous cycle of the track, performing all subtasks encountered along the path, and return to the start position.

### 4.2 Sub-Goals

| Priority | Sub-Goal | Trigger Condition | Success Condition |
| --- | --- | --- | --- |
| 1 | Follow the black line | Always active (base mode) | Robot stays on track |
| 2 | Navigate roundabout | Green or yellow patch detected | 4-step colour sequence completed, exits onto line |
| 3 | Avoid obstacle | Any proximity sensor < 0.20 m | Robot bypasses obstacle, reacquires line |
| 4 | Activate pickup | Blue marker detected by MiddleSensor | finish_line = True, BT flag item_picked set |
| 5 | Execute U-turn & deliver | Red marker detected after blue marker activation | 180° rotation completed, robot returns to blue marker |
| 6 | Complete mission | Robot reaches blue marker after U-turn | Simulation stops cleanly |
| 7 | Respond to vocal command | Stop/start/mode-switch broadcast received | BT blackboard updated, motor state changed |

## 5. Agent Architecture Design

The robot implements a three-layer hybrid architecture:

### 5.1 Deliberative Layer — Behaviour Tree

The BT is implemented using the py_trees library. It ticks once per actuation cycle. It maintains a blackboard (shared key-value memory) with the following keys:

| Key | Type | Description |
| --- | --- | --- |
| motion_mode | Enum (MotionMode) | Current driving mode |
| current_behaviour | Enum (VocalCMD) | Last vocal command state |
| vocal_cmd | Enum (VocalCMD) | Incoming vocal command |
| obstacle_detected | list[bool] × 3 | Per-sensor obstacle flags |
| obstacle_distance | list[float] × 3 | Per-sensor distances |
| detect_line | bool | Detects the line |
| detect_stop | bool | Detects the stop |
| change_beh | bool | Choose direction at interval |
| random_beh | string | Type of direction for movement |
| random_beh_dur | integer | Duration of current random behavior |
| obstacle_threshold | list[float] × 3 | Per-sensor thresholds |
| item_picked | bool | Whether pickup has been activated |
| item_reached | bool | Whether red marker has been hit |
| exit_reached | bool | Whether exit condition is met |

Motion Modes (MotionMode enum):

| Mode | Behaviour |
| --- | --- |
| LINE_FOLLOW | Standard line-following via 5 vision sensors |
| WALL_FOLLOW | Bang-bang obstacle bypass via proximity sensors |
| REACH_ITEM | Mission mode: colour detection + roundabout + U-turn logic |
| EXIT | Mission mode: navigate back to blue marker after U-turn |

### 5.2 Reactive Layer — Motor Dispatcher

sysCall_actuation() runs every simulation tick:

- Ticks the BT once (tree.tick_once())

- Prints BT state to console for debugging

- Checks current_behaviour — if STOP vocal command is active, halts motors immediately

- Reads motion_mode and dispatches to the appropriate motion primitive

- For WALL_FOLLOW: monitors safe-return conditions and switches back to the previous mode when the obstacle is cleared

### 5.3 Sensing Layer — Sensor Reader

readsensor.py runs in sysCall_sensing() (higher-frequency than actuation):

- Reads each vision sensor's packed packet; extracts index [10] (mean intensity)

- Applies threshold 0.5 to produce a binary line-detected boolean

- Reads each proximity sensor's distance and compares to threshold

- Publishes two broadcast messages per cycle: sensor_reading and proximity

- Streams all 5 sensor booleans to the CoppeliaSim graph for real-time visualisation

### 5.4 Message Bus

### All inter-script communication uses CoppeliaSim's sim.broadcastMsg() / sysCall_msg() mechanism, providing loose coupling between the sensor reader and the motor controller.

## 6. Testing Protocol

### 6.1 Unit Tests

| Test Case | Pass Condition |
| --- | --- |
| Line detected — centre | Robot moves forward at full speed |
| Line lost — left side | Robot rotates left to reacquire |
| Line lost — right side | Robot rotates right to reacquire |
| All sensors white (no line) | Robot holds last direction for 50 ms, then rotates |
| Proximity < 0.20 m (central) | Robot enters WALL_FOLLOW mode |
| Proximity cleared | Robot returns to previous motion mode |
| Blue marker in view | finish_line = True, mode switches to REACH_ITEM |
| Red marker in view after blue | 180° U-turn initiated |
| Roundabout green entry | 4-step steering sequence executed correctly |
| Roundabout yellow entry | Mirrored 4-step steering sequence executed |
| STOP broadcast received | Motors halt immediately |
| START broadcast received | Previous motion mode resumes |

### 6.2 Integration Tests

| Scenario | Steps | Expected Outcome |
| --- | --- | --- |
| Full lap, no obstacles, no markers | Start simulation; remove all optional objects | Robot completes full closed-loop cycle |
| Full lap with obstacles | Place bowl obstacles on track | Robot avoids each obstacle and returns to line |
| Pickup & delivery mission | Place blue marker then red marker on track | Robot activates pickup at blue, U-turns at red, returns to blue, simulation stops |
| Pickup activated, no red marker | Place only blue marker | Robot continues forward, completes full lap without reversal |
| Roundabout pass (green entry) | Ensure robot enters roundabout from green side | All 4 steering steps fire; robot exits correctly |
| Vocal stop mid-run | Broadcast stop_signal during run | Robot stops immediately; resumes on resume_signal |

### 6.3 Testing Environment Setup

- Run CoppeliaSim simulation in real-time mode

- Enable the /graph object to visualise sensor streams live

- Monitor console output for BT tick dumps and roundabout step logs

- Verify print() log statements match expected state transitions

## 7. Experimental Results

### 7.1 Sensor Calibration Data

Vision Sensor Binary Threshold:

- Threshold value: 0.5 applied to index [10] of the packed vision sensor packet (mean pixel brightness over the sensor FOV)

- Black line on white floor yields mean ≈ 0.0–0.3 → sensor reads True (line detected = dark)

- White floor yields mean ≈ 0.7–1.0 → sensor reads False (no line)

Proximity Sensor Calibration:

| Sensor | Threshold | Typical obstacle distance at detection |
| --- | --- | --- |
| CentralProximitySensor | 0.20 m | 0.10 – 0.19 m |
| LeftProximitySensor | 0.20 m | 0.10 – 0.19 m |
| RightProximitySensor | 0.20 m | 0.10 – 0.19 m |

Motor Calibration:

| Parameter | Tuned Value | Effect |
| --- | --- | --- |
| rad_mov | π rad/s | Forward cruise speed |
| rad_rot | π rad/s | Rotation speed |
| Internal sensor correction factor | −1.1011 | Fine-grained anti-drift |
| Outer sensor rotation factor | 1.5 | Aggressive recovery from edge |
| 180° turn speed | 14.0 rad/s | Completes ~π radians in 0.224 s |
| Roundabout steer base | π × 2.0 | Forward speed during roundabout curve |
| Roundabout steer diff | π × 1.4 | Speed differential between wheels |
| Wall-following base speed | 3.5 | Lateral obstacle bypass speed |

### 7.2 Sensor Data Stream

The CoppeliaSim /graph object records all five vision sensor streams per simulation tick. Sensor streams are offset vertically for readability:

- Left sensor: +0 (red)

- Middle sensor: +2 (green)

- Right sensor: +4 (blue)

- Internal Left: +6 (red)

- Internal Right: +8 (blue)

### 7.3 Acquired Data — Percept/Command Trace (representative excerpt)

t=0.00  sense=[F,F,F,F,F]  mode=LINE_FOLLOW   cmd=move_forward(π×2, π×2)
t=0.10  sense=[F,F,F,F,F]  mode=LINE_FOLLOW   cmd=move_forward(π×2, π×2)
t=0.55  sense=[F,F,T,F,F]  mode=LINE_FOLLOW   cmd=rotate_right(π×1.5, π×1.5)
t=0.70  sense=[F,F,F,F,F]  mode=LINE_FOLLOW   cmd=move_forward(π×2, π×2)
t=1.30  prox=[T,F,F] d=0.15 mode→WALL_FOLLOW  cmd=move_forward(steer_right)
t=1.80  prox=[F,F,F]       mode→LINE_FOLLOW   cmd=move_forward(π×2, π×2)
t=2.40  color=BLUE          mode→REACH_ITEM    finish_line=True
t=3.10  color=RED           cmd=start_turn_180 turning_180=True
t=3.33  turning_180 done    mode→EXIT          cmd=follow_line back
t=4.20  color=BLUE (return) exit_reached=True  sim.stopSimulation()

### 7.4 Videos

Placed the recorded simulation .mp4 file (one single) in docs/videos/ and link below.

### 7.5 Statistics with Graphs

Place generated statistic graphs .png files in docs/graphs/ and link below.

Performance Metrics Analysed:

- Line Tracking Error: Average deviation from the centre sensor over time.

- Obstacle Clearance Distance: Minimum distance maintained from obstacles during WALL_FOLLOW bypass.

- Mission Completion Time: Statistical breakdown of time spent in each mode (LINE_FOLLOW, WALL_FOLLOW, REACH_ITEM, EXIT) over 10 independent trials.

Mission Time Statistics Line Tracking Error

## 8. UML Diagrams

### 8.2 State Machine Diagram — Motion Modes

### 8.3 Sequence Diagram — Pickup & Delivery Mission

## 9. Non-Functional Requirements

### 9.1 Deployment — Dockerised Multi-Process

The simulation stack is split into independent processes:

docker-compose.yml
├── coppelia_sim      # CoppeliaSim headless instance
├── sensor_reader     # readsensor.py process (sensing)
├── motor_controller  # actuatormotor.py process (actuation + BT)
├── message_broker    # MQTT or ZeroMQ broker
└── gui_dashboard     # Real-time sensor GUI

Each process communicates exclusively via the message broker, replicating the sim.broadcastMsg() pattern in a real/distributed setting.

### 9.2 Message Broker

Message broker: ZeroMQ

Rationale:

- Decouples producers (sensor reader) from consumers (motor controller, GUI) — adding a new consumer requires no change to existing code

- Asynchronous, non-blocking delivery matches the real-time simulation tick model

- Topic-based routing maps directly to message id fields already used in the code (sensor_reading, proximity, stop_signal, etc.)

- Lightweight protocol suitable for embedded/real robot deployment

Topic mapping:

| sim.broadcastMsg id | ZeroMQ |
| --- | --- |
| sensor_reading | robot/sensors/vision |
| proximity | robot/sensors/proximity |
| stop_signal | robot/commands/stop |
| resume_signal | robot/commands/resume |
| switch_lf_signal | robot/commands/switch_lf |
| switch_ms_signal | robot/commands/switch_ms |
| avoid_cw | robot/commands/avoid_cw |
| avoid_ccw | robot/commands/avoid_ccw |
| avoid_auto | robot/commands/avoid_auto |
| turn | robot/commands/turn |

### 9.3 Logging System

The robot runtime is logged by the dump of every behaviour tree tick. And the robot sensors are logged using a graph. The graph is part of the logging system.

9.4 Shared Repository & Delivery

- Shared Repository: https://github.com/wojackbro/Optimised_Robot

- MS Teams Delivery: Upload code, docker-compose, videos, graphs, and the PDF design document to the private MS Teams group channel.

- Instruction file: README.md in the project root (install and test only).